#include <WiFi.h>

// Replace with your network credentials

const char* ssid = "Enter your wifi Name";

const char* password = "password";

WiFiServer server(80);

int ledPin = 2; // GPIO pin connected to LED

void setup() {

Serial.begin(115200);

// Initialize the LED pin as an output

pinMode(ledPin, OUTPUT);

// Connect to Wi-Fi

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting...");

}

Serial.println("Connected to Wi-Fi");

Serial.print("ESP32 IP Address: ");

Serial.println(WiFi.localIP());  // Print the IP address to the Serial Monitor

server.begin();

}

void loop() {

WiFiClient client = server.available();

if (client) {

String request = client.readStringUntil('\r');

client.flush();

// Check if the request is to turn the LED on

if (request.indexOf("/LED=ON") != -1) {

digitalWrite(ledPin, HIGH);  // Turn LED on

}

// Check if the request is to turn the LED off

if (request.indexOf("/LED=OFF") != -1) {

digitalWrite(ledPin, LOW);   // Turn LED off

}

// Send a response back to the client

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: text/html");

client.println("");

client.println("<!DOCTYPE HTML>");

client.println("<html>");

client.println("LED is now " + String(digitalRead(ledPin) ? "ON" : "OFF"));

client.println("</html>");

delay(1);

client.stop();

}

}