import cv2

import mediapipe as mp

import requests

# ESP32 IP address, replace with the actual IP address of your ESP32

esp_ip = "http://192.168.100.54"

# Function to send command to ESP32

def send_command_to_esp(command):

url = f"{esp_ip}/LED={command}"

try:

response = requests.get(url)

print(f"Sent {command} command to ESP32. Response: {response.text}")

except requests.exceptions.RequestException as e:

print(f"Failed to send command: {e}")

# Initialize MediaPipe Hands

mp_hands = mp.solutions.hands

hands = mp_hands.Hands(max_num_hands=1)

mp_draw = mp.solutions.drawing_utils

# Initialize camera

cap = cv2.VideoCapture(0)

def detect_gesture(hand_landmarks):

if not hand_landmarks:

return None

# Get key landmarks for gesture detection

thumb_tip = hand_landmarks.landmark[mp_hands.HandLandmark.THUMB_TIP]

index_tip = hand_landmarks.landmark[mp_hands.HandLandmark.INDEX_FINGER_TIP]

middle_tip = hand_landmarks.landmark[mp_hands.HandLandmark.MIDDLE_FINGER_TIP]

ring_tip = hand_landmarks.landmark[mp_hands.HandLandmark.RING_FINGER_TIP]

pinky_tip = hand_landmarks.landmark[mp_hands.HandLandmark.PINKY_TIP]

index_mcp = hand_landmarks.landmark[mp_hands.HandLandmark.INDEX_FINGER_MCP]

middle_mcp = hand_landmarks.landmark[mp_hands.HandLandmark.MIDDLE_FINGER_MCP]

ring_mcp = hand_landmarks.landmark[mp_hands.HandLandmark.RING_FINGER_MCP]

pinky_mcp = hand_landmarks.landmark[mp_hands.HandLandmark.PINKY_MCP]

# Measure distances from each fingertip to its respective MCP joint

def distance(a, b):

return ((a.x - b.x)**2 + (a.y - b.y)**2 + (a.z - b.z)**2)**0.5

index_finger_dist = distance(index_tip, index_mcp)

middle_finger_dist = distance(middle_tip, middle_mcp)

ring_finger_dist = distance(ring_tip, ring_mcp)

pinky_finger_dist = distance(pinky_tip, pinky_mcp)

# Simple logic: if all fingertip distances are small, hand is closed

if (index_finger_dist < 0.1 and

middle_finger_dist < 0.1 and

ring_finger_dist < 0.1 and

pinky_finger_dist < 0.1):

return "CLOSED"

else:

return "OPEN"

while True:

ret, frame = cap.read()

if not ret:

break

# Convert the image color to RGB

rgb_frame = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

# Process the frame to detect hands

results = hands.process(rgb_frame)

# Draw the hand annotations on the frame

if results.multi_hand_landmarks:

for hand_landmarks in results.multi_hand_landmarks:

mp_draw.draw_landmarks(frame, hand_landmarks, mp_hands.HAND_CONNECTIONS)

gesture = detect_gesture(hand_landmarks)

cv2.putText(frame, gesture, (10, 50), cv2.FONT_HERSHEY_SIMPLEX, 1, (255, 0, 0), 2)

# Send the gesture result to the ESP32

if gesture == "OPEN":

send_command_to_esp("ON")

elif gesture == "CLOSED":

send_command_to_esp("OFF")

# Display the frame

cv2.imshow("Hand Gesture Detection", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()